| NO | DATE | ENQUIRY | RESPONSE FROM TEAM |
| --- | --- | --- | --- |
|  | 23 November 2024 | Queries in whether performing water ponding test with supervise by Architect or his/her qualified representative to the Project ABC before issue for CCC since the master bathroom toilet leakage may cause collapse of ceiling at floor below and injure to the owner. The case repeated after 1 year first rectification (as informed from contractor by used grouting). As what I know the water proofing should have at least 5 years warranty. I wish it can have permanent solution as there are quite a lot of residence here facing the same issue and complaint to Maintenance team. | We refer to your email dated 25th November 2024 and have outlined the following points for your consideration based on the information provided: Water Ponding Test A water ponding test is a widely recognized method for verifying the effectiveness of waterproofing systems in buildings. However, it is not mandatory under Malaysian laws or regulations, nor is it explicitly required for the issuance of a Certificate of Completion and Compliance (CCC). The Architect’s role concerning the CCC is primarily to certify that the Works were supervised and constructed in compliance with the Approved Plans. Supervision of the Water Ponding Test The water ponding test is typically included in the project's technical specifications, which form part of the construction contract. This document details the methodology, scope, and responsibilities, including identifying key personnel overseeing the test. Waterproofing Warranty Waterproofing warranties are an industry practice, not a legal requirement. The terms and conditions of such warranties are usually determined at the discretion of the Developer, Architect, or other specifiers, depending on the project needs. If no specific warranty is provided, the Defects Liability Period (DLP) in the Sale and Purchase Agreement (SPA) will apply. We advise reaching out to the Developer for clarification on any waterproofing warranty applicable to your case. Rectification of Defects If the DLP is still valid, we advise contacting the Developer to address the defects. Refer to Clause 30 of the SPA, which outlines your rights during the DLP. Should defects persist after the DLP has expired, you may consider seeking legal advice or filing a claim with the Tribunal for Homebuyer’s Claims under the Ministry of Housing and Local Government (KPKT). Maintenance Considerations Building materials, including waterproofing systems, are subject to wear and tear. While no waterproofing solution is permanent, regular monitoring and maintenance are essential to minimize issues over time. We trust the above has been of assistance. |
|  | 6 November 2024 | The purchaser would like to inform the management of Developer A that we are not happy and very unsatisfied with the finished unit that was handed over to us. This is due to multiple design failures and other defects as mentioned below. - There are multiple design issues / failures. - We believe that substandard materials were used. - Material specification and actual design specifications the architect was not provided to buyers. Why? - Defects - mostly completed via superficial/ artificial/ temporary solution - with very high chances of recurring defects within 6 months to 1 years’ time. We are seeking / asking and demanding for a joint inspection together with the Developer, Contractor, Architect, MDBK (Jabatan Perancangan) and a senior member from Malaysian Institute of Architects . We will not agree to a joint inspection on the defect rectification, if any of the mentioned parties do not participate. We believe that all parties must take responsibility and join this session. Issues and response from Developer (numbers) and Buyer (Point form) - The type of door used for all of the rooms is High Density Fibre with High Moisture Resistance (HDF-HMR). To date we have rectified all defective doors in your unit. - Please share the specifications of the door that was used in our units? - Kindly share the documents (purchase of door materials - specifications) to ensure the correct type of materials was used. - This is a serious design failure - Please indicate where in the UBBL. Where does it state that you can build in this manner? I have gone through the UBBL and can't find any specific clause regarding this. - What design was submitted to MDBK. We are also not sure how MDBK approved this design? - Did MDBK visit any of the units? If so, when did they visit and what was the conclusion of the visit? - The type of material used for the piping systems. For the water main it is HDPE pipes in accordance with Air Selangor Klang requirements. From tapping to the water meter, stainless steel pipe is used. From the water meter to the storage tank and to all the outlets the material used is HDPE pipe. All material used is in compliance with SPAN. - Kindly share the documents (purchase of materials - specifications) to ensure the correct type of materials was used. - Kindly share the documents (Air Selangor Klang requirements). - Kindly share the documents (SPAN - requirements). - How do we check if the correct materials were used? - We have rectified the water pressure issue. Our team re-tested the outlets again on 25 October 2024 and there is no longer a problem with the water pressure. (Please refer to the videos in attachment A to C). - On 27th October 2024 - We found there is still insufficient water pressure. (Refer Picture and video) - The pressure from the main to the kitchen pipe is very slow. - The pressure from the tank is very poor at the master bedroom bathroom. - The pressure from the tank is very bad to all the other outlets. - Propose to invite Air Selangor (SPAN) for a site visit together with Developer and Buyers - If everything is in order then there is no reason for SPAN to decline a joint site visit right? - On the flood mitigation pond, we will respond once our Civil and Structural Engineer revert to us. We will try to respond within 7 days. - This issue must be resolved ASAP. - How to prevent mosquitoes from laying eggs in the flood mitigation pond? - Who will be responsible for cleaning the mitigation pond? or to ensure no mosquitoes enter and lay eggs? - If KKM issues a fine/ instructs further actions (mortify/ change) - who will be responsible? - This issue has the potential to be dragged on and it involves KKM and other government bodies. - There is now water supply to the garden outlet as it has been rectified. - We did not find any pipes or outlets for the garden area? - Please share the pictures - showing pipes were installed. - On the bathrooms, it is designed in compliance with Uniform Building By-Law (UBBL) and in accordance with the approved building plan by Majlis Bandaraya Di Raja Klang. As per our contractual obligation in the SPA we will rectify any defective or faulty doors. - From my picture - The water from the shower will touch the door in the bathroom? - Will the door in the bathrooms be damaged from the shower water/ splashing? (matter of time) - Your side (Contractor/ Developer) will change the damaged doors (for the duration of the defect period) correct? - What will happen if the door is damaged by water splashing from the shower after the defect period? - This is a serious design failure - Needs urgent rectification/ solution. - As a practicing safety and health officer, I am familiar with UBBL. Please indicate where in the UBBL does it state that you can build in this manner? I have gone through the UBBL and can't find any specific clause regarding this. - We are also not sure how MDBK approved this design? Did MDBK visit any of the units upon completion? - If so, when did they visit and what was the conclusion of the visit? Did they detect any failures? Where is the report? - We inspected the car porch and did not detect any sinking. Perhaps you can demarcate or indicate the area of the car porch in which you observe the sinking to enable us to inspect and rectify if required. - We will demarcate and indicate the area of the car porch that is showing signs of sinking/ water ponding. We have had discussions with majority of the buyers of XXX Residences and we found that, all the buyers have the same issues as mentioned below. - Car Porch sinking/ water ponding - Insufficient water pressure - Reappearing cracks (walls and slabs) - Electrical - improper connection (major tripping/ no supply) - No power supply to the power points in the hall and ground floor room. - Electrical - current coming from the earth and life wire at the same time - causing tripping. | We refer to the enquiry form received on 6th November 2024. It seems that the list of questions was intended for a developer, with no specific enquiries directed to the PAM Practice Bureau (PB). Nonetheless, based on the information provided, we wish to offer our responses as follows: Direct Engagement with the Developer We noted your engagement with the developer and advise that you continue to address any defects, ensuring that all necessary rectifications are completed within the Defects Liability Period (DLP). Should you require further documentation or clarification on specific defects, we recommend formally requesting this information from the developer in writing. Independent Inspection Option If you have concerns about the adequacy of the developer's response to defects or if you prefer an independent assessment, we recommend engaging a qualified and independent professional to inspect the units. Should you encounter difficulty in identifying such a professional, you may consider contacting Architect Centre Sdn Bhd (www.architectcentre.com.my), who could assist you with the inspection process. Tribunal for Homebuyer’s Claims If your property is governed by the standard Sales and Purchase Agreement under the Housing Development (Control and Licensing) Act 1966 (Act 118) & Regulations (HDA), you can file a complaint with the Tribunal for Homebuyer Claims under the Ministry of Housing and Local Government (KPKT) if the developer fails to address defects adequately. This tribunal offers an efficient, cost-effective platform for resolving property defect disputes. Request for Joint Inspection In response to your request for a joint inspection involving the Developer, Architect, Contractor, Local Authority, and a senior representative from the Malaysian Institute of Architects (PAM): please note that PAM is a private institute of architects in Malaysia. It does not possess statutory powers of enforcement and therefore would have no legal standing to attend any joint inspections requested by yourself, the developer, architect, or local authority. Local Authority Compliance We regret to inform you that we cannot offer guidance regarding the local authority's stance on compliance with design regulations or approvals. For precise information on design compliance or other regulatory concerns, we recommend contacting the appropriate local authority directly. We trust the above has been of assistance. |
|  | 29 October 2024 | I, a resident at Project ABC, located at Kuala Lumpur would like to seek your assistance in the investigation on the integrity of our building. This past Sunday, 20 October 2024, a piece of our facade wall had fallen off the 10th floor landing near the basement entrance. Luck would have it that this incident happened at 3:30am resulting in no fatalities or injuries. In addition the facade walls, we have also been having issues with the water heater wiring catching fire. This has been on-going for months and we have yet to receive any feedback from the developer on how to mitigate or any intentions to rectify the matter. This issue begs to question the integrity of the remaining structure. The developer is in the midst of repair works however as residents, we would like to request for a formal investigation to be conducted on the facade. I have attached herewith a few photographs of the incident. We hope that your esteemed establishment would be able to help us bring justice. | Thank you for your inquiry dated 29th October 2024. We have noted the safety concerns you raised regarding your residence. Please be informed that the PAM and its Practice Bureau are focused on providing feedback on practice-related inquiries and do not offer inspection or investigative services related to building integrity. Nevertheless, below is our feedback on the issues highlighted in your inquiry based on the limited information provided: If your residence is still within the Defects Liability Period (DLP) under the Sales and Purchase Agreement (SPA), we recommend reviewing your entitlements as per the DLP provisions. It may also be beneficial to consult a legal advisor to understand your rights under the SPA. Nevertheless, we advise that you continue engaging with the Building Manager and Developer to ensure that all lines of communication are maintained. We acknowledge the records you attached regarding balcony glass issues, burnt power sockets, and delaminated wall finishes. Given the potential safety risks, we recommend engaging an independent professional inspection service to conduct a thorough assessment. Should you have difficulty in finding such a service and require assistance in locating a qualified inspector, you may consider contacting Architect Centre Sdn Bhd (https://www.architectcentre.com.my/ or Cik Naza Izana at 019-399 4869), who may be able to assist in providing such a service. In summary, seeking advice from both a legal advisor and a qualified professional can provide clarity on your rights and guide you on the appropriate steps to address these concerns effectively. We hope this information has been helpful. |
|  | 4 July 2024 | Clarification on the Eligibility of Peninsular Malaysia-based ECPs to Perform Authority Submissions in Sabah and Sarawak. | We refer to your enquiry received on 4th July 2024. Based on the information provided, please find our response below: Your query pertains to whether an Engineering Consultancy Practice (ECP) with a registered office in Peninsular Malaysia can carry out Authority Submissions in Sabah or Sarawak. Generally, all submitting persons must be registered with the relevant state authorities to practice and make submissions in Sabah and Sarawak. Each state has unique regulations and registration requirements, which may include provisions for temporary or special registration. We recommend consulting with the relevant state authorities in Sabah and Sarawak to understand the specific registration processes and requirements for engineers. Besides, we suggest inquiring with the respective state's immigration authorities regarding the requirement for obtaining a work permit. This permit is essential to comply with Sabah's or Sarawak's immigration regulations, ensuring legal authorisation for employment within these states. We also advise contacting the state governments, local councils, and local branches of the Board of Engineers Malaysia (BEM) and/or the Association of Consulting Engineers Malaysia (ACEM) to obtain accurate and up-to-date information tailored to your specific situation. We acknowledge that a similar query has been directed to the Board of Engineers Malaysia. We trust that your due diligence will be supported by the response from the Board of Engineers Malaysia, providing valuable and informative guidance on this matter. We hope the above information has been of assistance. |
|  | 6 June 2024 | One of the owners for a YYY condominium unit situated in Cheras, Selangor, developed by XXX Bhd. Recently, there is an uproar from the group of owners in regards to an official statement shared by YYY project's architect firm, ZZZ Architect Sdn Bhd to address the vastly discovered hollow tiles across the residential units. I have attached the official statement from ZZZ Architect Sdn Bhd, which claims that tile hollowness is not considered a defect, aiming to shield the developer from providing any sort of rectification works for the group of owners. Since most of the owners are commoners and have no in-depth knowledge in this matter, we hope to seek for PAM's view in regards to the official statement shared by these professional architects. In layman's understanding, hollow tiles are definitely a defect for us. Thank you in advance. | We refer to your request for our opinion as received on 6 June 2024: When the Architect in question and yourselves refer to "tile hollowness", we shall assume that what is meant is the presence of a hollow or drumming sound when the surface of the tile is tapped. The above can occur for several reasons including but not limited to: - Entrapped air or voids in either the setting bed or slab below, causing one part of the floor to sound differently than another; - Possible separation of the waterproofing membranes installed between a slab and the bedding material; - Possible vaulting of the tile due to a lack of room for expansion; - The shrinking of the tile bedding as it dries out. We are of the opinion though that generally, a tiled surface is meant to: - provide an aesthetically acceptable surface; - allow for the drainage of water or moisture; - assist in preventing the penetration of water into the underlying wall and floor; - provide a surface which allows for safe habitation and passage; - provide sufficient support for the activities for which the space is designed for. If the tiling is able to achieve the above, the Architect's view that "A hollow sounding tile may not mean that the tile is defective and needs to be replaced" is not incorrect. Nevertheless, depending on the severity of the hollow sound relative to the tiled surface, this may be indicative of the possibility of insufficient support or adhesion below the tile; insufficient support or adhesion may result in the possible cracking of the tile when it is stressed/when a load is imposed upon it or the tile coming loose or falling off. An elevated probability of tiling becoming defective is not the same though as the tiling being defective. It must also be noted though, that the cracking or coming loose of tiles may also occur even when there is no hollow sound detected. Should you have any reservations on the ability of your tiling to perform to accepted standards, we suggest that a qualified and independent professional be engaged inspect the tiles. Should you have difficulty in finding such a professional, you may consider contacting Architect Centre Sdn Bhd (https://www.architectcentre.com.my/) as they may be of assistance in providing such a service. We hope this clarifies the situation and assists you in taking the necessary steps to address your concerns. |
|  | 4 January 2024 | Project 9 blocks of Service Apartment Block sitting on 4 level car parks podiums and 2 levels of sub basement consist of 1800 units. - Phase 1 - 1000 units with clubhouse & swimming pool and other facilities. - Phase 2 - 800 units with swimming pool and other facilities. Both Phases are under 1 master land title and governed by HDA. Current status: Phase 1 - 1000 unit (with all required car parking lots), all facilities - Building Plan Approved with clear demarcation of boundary by Local Authority. Phase 2 - 800 unit (with all required car parking lots), all facilities -- Building Plan Approved with clear demarcation of boundary by Local Authority. Developer has applied 2 separate Advertising Permit (Phase 1 & 2), pending approval. Developer intended to launch Phase 1 first and phase 2 will be 1 year after that with different completion date. My question is for Phase 1, after all the works has been completed and complied to Approved Building Plan as per Phase 1, can architect issue Borang F for this phase 1 instead of Borang F1? | We refer to your e-mail with regards to the Certificate of Completion and Compliance. As we have no details regarding the project title of your project submission, we assume the project is submitted under one (1) building plan. According to Street Drainage Building Act, Section 70 (21), the principal submitting person (PSP) is required to supervise the erection of the building to ensure that the erection is in conformity with the approved plans and other requirements. In this case, the approved building plan is only one (1) submission even though there are two (2) phases. Since there is only one (1) Building Plan, only one (1) CCC can be issued out. As such, we are of the opinion that CCC (Certificate of Completion and Compliance) or Borang F cannot be issued unless the whole project is completed. Further to this, Housing Development Act Section 3 clearly stated Certificate of Completion and Compliance for housing development does not include Partial certificate of completion and compliance (Form F1). Therefore, issuance of Form F1 for phase 1 will not help the developer to issue vacant possession earlier if he wishes to do so. A proficient resolution to address this matter involves pursuing an amendment to the building plan, thereby bifurcating the project into two phases and corresponding building plans. We recommend initiating communication with the relevant authority to seek clarification on the submission process and associated details. We hope the above reply to your query is satisfactory. |